NOTE to document editor: Proficiency in Microsoft WORD is a prerequisite to authoring or editing a new or revised NENA document.

Please note the following document conventions:

- All explanatory text shown in red text or in blue boxes IS TO BE REMOVED before submitting the document for a WG approval ballot.

- All blue text will be managed by the CRM team.

- Black brackets [ ] indicate fields the editor fills in.

- All black text is boilerplate text and is NOT to be changed (unless the text is in brackets [ ], which indicates that the editor is to make necessary changes).

  - If a section is noted as optional and is not needed, delete the section title and black text.

NENA [NG911 Management Considerations for EIDO Interoperability]

ABSTRACT: [one or two sentences describing the content of the document]

The CRM team will provide the remainder of the information on this page.

NENA-INF-NNN.N.Na-YYYY

NENA Board of Directors Approval: 	Month DD, YYYY

Next Scheduled Review Date: 	Month DD, YYYY

Prepared by:

National Emergency Number Association (NENA) [insert the applicable Committee, Subcommittee, and/or WG names]

This DRAFT document is not intended for distribution beyond the groups developing or reviewing the document. The document is also not intended to be used or referenced for development or procurement purposes until final publication. All draft material is subject to change, and it is possible that the document itself may never be approved for publication.

NOTE to document editor: The Table of Contents defaults to showing three heading levels. If the Working Group determines that it is important to show a fourth heading level, customize the Table of Contents using the following instructions:

- Click within the Table of Contents, and select the References tab.

- Select Custom Table of Contents from the Table of Contents drop-down menu.

- Set Show Levels to 4 under General.

- Select OK.

Remove the List of Tables and List of Figures if they are not useful for the document.

TABLE OF CONTENTS

EXECUTIVE OVERVIEW	iv

NENA INFORMATION DOCUMENT NOTICE	v

Document Terminology	vi

Unit of Measurement	vi

Describing Artifacts	vi

NENA Intellectual Property Rights (IPR) and Antitrust Policy	vii

Reason for Issue/Reissue	vii

1	[HEADING TITLE]	1

1.1	[Heading Title]	1

1.1.1	[Heading Title]	1

2	Dependencies	1

3	Impacts and Considerations	2

4	Recommendations for Additional Development Work	2

4.1	[Future Work Item]	3

4.1.1	[Description]	3

4.1.2	[Recommended Assignment]	3

5	Abbreviations, Terms, and Definitions	4

6	References	5

7	Appendices	8

7.1	Appendix X—[Appendix Title]	8

ACKNOWLEDGMENTS	9

Special Acknowledgments	9

GENERAL DOCUMENT EDITING HELP	11

LIST OF TABLES

Table 4-1 Future Work for Consideration	3

Table 7-1 Example Table with 3 Headings (3 columns)	14

Table 7-2 Example Table with 4 Headings (4 columns)	14

LIST OF FIGURES

Figure 7-1 Effect of Geocoding Addresses against Actual Address Ranges	15

Figure 7-2 NENA i3 NG9-1-1 Reference Architecture	15

EXECUTIVE OVERVIEW

NOTE to document editor: Every document includes an Executive Overview. The Overview should be unique and not just be cut-and-pasted from another section. In most cases, this section should be no more than one (1) page and have no subsections. This section does not contain normative language. It should provide a brief summary of the entire document which can be developed by answering the following questions:

- What is the goal of the document/what problem or issue is it trying to solve?

- What is the background of the issue?

- Who is affected by the issue and the solution?

- Are there any other important things that readers of this document should know or be aware of?

If additional information is required, the Working Group may choose to create a Background or Introduction section at the beginning of the main body of the document. This can also be included as a Purpose and Scope section or a Benefits section that may have existed in earlier versions of the document’s Executive Overview if the Working Group determines it is not appropriate to continue to maintain these subsections here.

This document describes the management of the information to governing the movement of Emergency Incident Data Objects (EIDOs) incident data from one part of the Public Safety Answering Point (PSAP) / Emergency Communications Center (ECC) to other parts of the PSAP/ECC or to other PSAP/ECCs or ECCs using an Emergency Incident Data Object (EIDO). The core scope of this document covers:

1. What is an EIDO and when is one sent;

2. What data can be put into an EIDO;

3. Who is authorized to send and receive an EIDO - criteria used to determine;

4. What best practices should be applied to EIDO management; and

5. Are interagency agreements necessary to exchange EIDOs outside of the NG911 system.

Data that is available today in evolving smart technologies is not always receivable by today’s PSAP/ECCs, and today’s PSAP/ECC capabilities commonly are very limited in interoperable data sharing except through manual or customized automated processes that are unique to each PSAP/ECC. NG911 technology addresses these limitations through standardized functional elements and interfaces which are policy driven application processes. Each agency’s NG911 system will function based on the agency’s data management policies. Currently NENA has no standard or best practice for PSAP/ECC Managers and the local Authority Having Jurisdiction (AHJ) in the development of their data management policies.

It is imperative that NG9-1-1 Authorities and PSAP/ECC Managers understand that data management, through the policies they write, will drive their system’s interoperability. They must also understand the corresponding responsibilities, including risks and risk mitigation through interagency agreements defined in policies, as well as security policies and processes.

Our approach in this document is to provide PSAP/ECC Managers and AHJ with an understanding of NG911 incident data, the functional role of the Emergency Incident Data Object referred to as EIDO, and the NG911 environment; along with recommendations for best practices in policy development considerations as they impact the critical tasks of dispatch operations. Careful policies will be key to successful implementation of NG911 services and their impact on the PSAP/ECC, interoperability with other PSAP/ECCs, while also addressing critical data protection needs.

Through good data management a PSAP/ECC can redefine operational tasks which impact the workload yet expand the level of emergency service they provide. Local responsibility must address both data protection and improving service.

The scope limits of this document shall serve as a basis for PSAP/ECC management NG911 implementation and the sharing of data as it directly applies to managing the emergency incident response. It is beyond the scope of this document to provide a standard for 9-1-1 data sharing during large scale disaster recovery, among various Government services agencies, with the media, or any non-public safety entities that aren’t ancillary to the emergency incident. While these services provide societal necessities, data sharing interoperability between the PSAP/ECC and these services has been identified for future work. Many other NG911 considerations outside of the scope of this document can be found in related NENA, APCO, or other Standards Developing Organizations (SDO) published documents.

EXAMPLE:

This document outlines the crucial aspects of managing tasks within the Public Safety Answering Point (PSAP) environment, particularly as the industry transitions to Next Generation 9-1-1 (NG9-1-1). The document emphasizes managing critical tasks, addressing performance issues, and preventing task overload. This is particularly important as new systems and technologies are introduced with NG9-1-1.

PSAP managers are expected to handle staffing, retention, training, and quality assurance, often with limited resources. These challenges are projected to intensify with the ongoing migration to NG9-1-1. Stakeholders in 9-1-1 systems must understand the issues related to task overload and how it affects service delivery. This understanding is crucial for effective 9-1-1 operations.

The document’s recommendations are grounded in industry best practices, research, and experience. They aim to improve the quality and delivery of 9-1-1 services.

NENA INFORMATION DOCUMENT NOTICE

NENA: The 9-1-1 Association improves 9-1-1 through research, standards development, training, education, outreach, and advocacy. Our vision is a public made safer and more secure through universally-available state-of-the-art 9-1-1 systems and better-trained 9-1-1 professionals. Learn more at https://www.nena.org.

This Information Document (INF) is published by the National Emergency Number Association (NENA) as an information source for those that support emergency calls such as 9-1-1 System Service Providers (9-1-1 SSPs), network interface vendors, system vendors, telecommunications service providers, and 9-1-1 Authorities. It is not intended to provide complete design or operation specifications or parameters or to assure the quality of performance for systems that process such equipment or services.

NENA reserves the right to revise this Information Document for any reason including, but not limited to:

Conformity with criteria or standards promulgated by various agencies,

Utilization of advances in the state of technical arts,

Reflecting changes in the design of equipment, network interfaces, or services described herein.

This document is an information source for the voluntary use of public safety communication centers, such as 9-1-1 Public Safety Answering Points (PSAPs). It is not intended to be a complete operational directive.

It is possible that certain advances in technology or changes in governmental regulations will precede these revisions. All NENA documents are subject to change as technology or other influencing factors change. Therefore, this NENA document should not be the only source of information used. NENA recommends that readers contact their 9-1-1 Authority and 9-1-1 SSP representative to ensure compatibility with the 9-1-1 network, and their legal counsel to ensure compliance with current regulations.

Patents may cover the specifications, techniques, or network interface/system characteristics disclosed herein. No license is granted, whether expressed or implied. This document shall not be construed as a suggestion to any manufacturer to modify or change any of its products, nor does this document represent any commitment by NENA, or any affiliate thereof, to purchase any product, whether or not it provides the described characteristics.

By using this document, the user agrees that NENA will have no liability for any consequential, incidental, special, or punitive damages arising from use of the document.

NENA Development Group (NDG) Committees have developed this document.

Recommendations for changes to this document may be submitted to:

National Emergency Number Association

1700 Diagonal Rd, Suite 500

Alexandria, VA 22314

or crm@nena.org

Document Terminology

NENA information documents do not introduce new normative language as defined in RFC 2119 [2] and later updated by 8174 [3].

Unit of Measurement

NOTE to document editor: The NENA Development Group Operational Procedures, ADM-001, Section 6.1 Appendix A—NENA Metric Policy states that Units of the International System (SI) and/or United States Customary System (USCS or USC) may be included as appropriate. The editor MUST identify the appropriate unit of measurement. Multiple options are available to identify the unit of measurement: US, Metric, both US and Metric, and Not applicable. If no measurement is included in the requirement, the appropriate unit of measurement is “Not applicable.” The editor should select one of the following statements to include in the requirement.

Where a measurement is found in the document, the unit of measurement is [US, Metric, or both US and Metric]. When a measurement is used in the document, the unit of measurement is identified.

No measurement is used within this requirement; therefore, the unit of measurement is “Not applicable”.

Describing Artifacts

NOTE to document editor: If the main body of your document includes Artifacts, include this "Artifacts” section and the text immediately below; otherwise, delete this section and, if not referenced elsewhere in the document, remove reference [4], [5], and [6] in Section 5 References.

There are Artifacts [1] in this document, as defined in NENA Rules for Use of Version Control Repositories, NENA-ADM-012 [4]. Artifacts defined in NENA documents, such as Representational State Transfer (REST) interfaces and JavaScript Object Notation (JSON) objects, usually use OpenAPI [5] as their formal description. In some cases, XML is used [6].

In the description of the interface or object in NENA documents, a table may appear that describes the parameters to the interface and the data objects. These tables are informative; the Artifact file is the normative documentation of the interface and data objects unless otherwise specified in the document. If there is a discrepancy between the table and the Artifact file, the Artifact file controls unless otherwise specified in the document.

In the tables, there is a column named "Required." The following three (3) values may occur in this column:

- A “Yes” means the parameter or member MUST be present. It MUST appear in the “required” section of the Artifact file.

- A “No” means that the parameter or member MAY be present. It MUST NOT appear in the “required” section of the Artifact file.

- A “Conditional” means that the presence of the parameter or member is conditional. This value alerts the reader that a business rule is specified, which controls the presence of the parameter or member. It MUST NOT appear in the “required” section of the Artifact file. This is because notation formats like YAML do not provide a way to support a conditional business rule.

Business rules are contained in this document for associated Artifacts. These business rules MUST be followed in implementing the standard.

NENA Intellectual Property Rights (IPR) and Antitrust Policy

The user's attention is called to the possibility that compliance with this document may require use of an invention covered by patent rights. By publication of this document, NENA takes no position with respect to the validity of any such claim(s) or of any patent rights in connection therewith. If a patent holder has filed a statement of willingness to grant a license under these rights on reasonable and nondiscriminatory terms and conditions to applicants desiring to obtain such a license, then details may be obtained from NENA by contacting the Committee Resource Management team at crm@nena.org.

Consistent with the NENA IPR and Antitrust Policy, available at https://www.nena.org/ipr, NENA invites any interested party to bring to its attention any copyrights, patents or patent applications, or other proprietary rights that may cover technology that may be required to implement this Information document.

Please address the information to:

National Emergency Number Association

1700 Diagonal Rd, Suite 500

Alexandria, VA 22314

or crm@nena.org

Reason for Issue/Reissue

NOTE to document editor: For all revisions to the initial document, the WG/editor will need to provide a synopsis of the Reason for Issue/Reissue. While it should be as brief as possible, it should permit the reader to have a good understanding of what has changed in each revision. If a major rewrite was performed, a statement to that effect is appropriate instead of trying to list all the changes. The CRM team will insert the official document number and Approval Date.

If the document modifies any git repository, it shall be recorded here with the version of the repository this document creates or modifies, including the Artifact version and document version tags.

NENA reserves the right to revise this document. Upon revision, the reason(s) will be provided in the table below.

| Document Number | Approval Date | Reason For Issue/Reissue |
| --- | --- | --- |
| NENA-STA-[NNN.1-YYYY] | [Month DD, YYYY] | Initial Document |
| NENA-STA-[NNN.2-YYYY] | [Month DD, YYYY] | [Example: Major rewrite; updated URLs] |
| NENA-STA-[NNN.2.1-YYYY] | [Month DD, YYYY] | [Example: Minor update; Added widget escape procedure] |
| NENA-STA-[NNN.2.1a-YYYY] | [Month DD, YYYY] | [Example: Fixed unintentional inconsistency in widget processing] |
| NENA-STA-[NNN.3-YYYY] | [Month DD, YYYY] | [Example: Modifications to section(s) (list the sections)] |
| NENA-STA-[NNN.4.1-YYYY] | [Month DD, YYYY] | [Example: Log event added to Logging Service in Section X.Y; authoritative version at https://xyz] |
| NENA-STA-[NNN.4.1a-YYYY] | [Month DD, YYYY] | [Example: corrections to schema in Appendix ABC; authoritative version at https://xyz] |

# Emergency Incident Data Object (EIDO) Definition and Introduction To The management of Inter-operable NG9-1-1 SystemS

NOTE to document editor: Replace “Heading Title” here and in each section with an appropriate title that describes the subject matter of that section.

## An Introduction to the EIDO

NOTE to document editor: Document topic-specific section/subsection headings and descriptive text are to be included in this section. The following instructions provide greater detail. However, if the editor has questions regarding how to use this template, they should contact the WG Chair(s), the Committee Chair(s), or the Committee Resource Management Team for assistance.

The Heading Title (that the authors of this document will provide above) serves as the beginning of the specific detailed portion of the document. The authors of this document would add any necessary headings within this section to complete this document. These sections can be as long as needed to support the document topic. Editors are advised to use as few heading levels as possible for readability, preferably not to exceed Heading Level 4 and to include no more than Heading Level 6. This section can also be broken up into multiple Heading 1 levels if the content organization is better served than having everything in one section that uses several nested heading levels.

In addition, the document editor should

- be consistent with the use of bullets and item numbering,

- identify heading levels for all sections and subsections by using the appropriate WORD Styles,

- use WORD’s built-in “cross-reference” function to indicate where the reader can find associated “References” that are listed below in Section 56 References or for other document navigation purposes, and

- upload the source files for all diagrams, images, and illustrations (e.g., Visio, Illustrator, PowerPoint) into the NENA Workspace for future editing.

Additional information on these and more editing pro-tips can be found in the General Document Editing Help section at the end of this Template.

In a NG9-1-1 environment Emergency Incident information is transferred between parties seamlessly through a standard interoperable method. The mechanism that enables this is the EIDO, which is a JSON-based (JavaScript Object Notation) data structure used to share emergency incident information between and among authorized entities and systems. Think of an EIDO as a current snapshot of the information known of an incident at the moment in time when it is sent to another entity based on your agency’s data sharing policies. NENA has adopted the JSON-based EIDO for sharing incident information among authorized Next Generation 9-1-1 (NG9-1-1) entities and systems.

Throughout this document emergency incident information will be referenced as Incident Data.  An incident is a real-world occurrence, or event, for which one or more calls may be received. It will be the responsibility of the Agency to determine Incident Data sharing policies related to which agencies it will share Incident Data with. Once data sharing policies are established, the agencies functional elements will be configured to reflect them and automatically share Incident Data.

“A functional element is a set of interfaces and operations on those interfaces to accomplish a task.  Within i3 there are three primary functional elements:  Call Handling, Incident Handling and Dispatch.  A Functional Element can subscribe to other Functional Elements, both internally within an agency and externally to Functional Elements belonging to outside agencies.  A subscriber may subscribe to either all incident EIDOs or a subset thereof, a single incident’s EIDOs or to all EIDOs related to a specific resource.  What information a subscriber can see is dependent on the hosting agencies Data Right’s Management (DRM) policies.”

You may want to reference the Glossary definition of FE (Functional Element) for more detail.
https://kb.nena.org/wiki/FE_(Functional_Element)

## History

In E9-1-1 systems, caller information is retrieved at a PSAP from an ALI (Automatic Location Identification) database.  The call handling controller then delivers this information to CAD (Computer Aided Dispatch) systems over a serial interface described in NENA-STA-027.3-2018.  The information in the ALI response and in the data transferred to the CAD system included caller information such as location, caller name, and responsible police, fire, and EMS services for that location.

NENA intends the use of EIDO to relay emergency incident data between functional elements within a PSAP, between PSAPs, between PSAPs and other external entities such as law enforcement agencies, fire departments and emergency medical services (EMS). Unlike legacy E9-1-1 systems, which require a telecommunicator to verbally notify agencies of changes in incident status, the standard EIDO format allows NG9-1-1 systems to notify agencies of incident status changes using machine driven automatic methods.

## Applicable Functional Elements

In legacy PSAPs, we have two large systems we generically call “CPE” and “CAD”.  CPE handles the 9-1-1 call, and CAD handles incidents and the response to those incidents.

Within a PSAP, all NG9-1-1 compliant systems that deal with incident data use EIDOs as the standard way to express what one system or agency knows about an incident.  By using EIDO as the common way to exchange incident data, we get a level of interoperability between disparate systems. The EIDO is the data format of the information the Call Handling FE provides the Incident Handling FE about a 9-1-1 call and the data format of the information the Incident Handling FE tells the Dispatch FE the data that was collected and entered by the call taker and received with the call.

To standardize the exchange of information in the computer based NG9-1-1 environment, Functional Elements have been defined.  There are four Primary Functional Elements currently defined:  Call Handling, Incident Handling, Dispatch and IDX (Incident Data eXchange).  Multiple Functional Elements may be combined in a single vendor product or service or may be provided by different vendors.  All Functional Elements should interoperate across various vendor products using EIDOs.

IDX is a Functional Element that facilitates the exchange of EIDOs among other Functional Elements both within and external to an agency. Since these exchanges may be both internal and external, agencies must be aware of how this functionality will impact data flow when forming their policies on EIDO transactions.  The IDX will implement the EIDO management policies, for external entities so the policy will need to consider the destination in any data management considerations, which is the primary focus of this information document.

## What Data Can Be Put into an EIDO

‘The components of EIDOs is are populated using the JSON Schema as defined in NENA-STA-021.  The data components of an EIDO Emergency Incident Data Object Data Components include, but are not limited to:

- Agency Data

- Split/Merge Data

- Link Data

- Incident Data

- Call Data

- Call Back Data

- Updated Call Back Number

- Dispatch Data

- Disposition Data

- Notes Data

- Person Data

- Vehicle Data

- Location Data

- Additional Data

- Emergency Resource Data

- Alarm and Sensor Data

For more details regarding the data components of EIDOs please refer to NENA-STA-021.

Historical data is stored in the logging service. All details, including event history, regarding the incident that were embedded in previous EIDOs will be maintained by the logging service. EIDOs represent the current state of an incident. Detailing the logging service is outside the scope of this document.  Refer to NENA-STA-10.3b-20214 for further information on the logging service.

INSERT DIAGRAM IN SECTION 2 OF DEPENDENCIES ?

Note:  IDX as well as the process for Non-Public Safety Entities has not yet been defined.

## Exchanging Information in the NG9-1-1 Environment

# (DEPENDENCIES)

## Who is Authorized to Send and Receive EIDOs?

Agents within a PSAP as well as external agencies electronically subscribe to the Incident to maintain situational awareness of incident status. The PSAP should create a Data Rights Management (DRM) policy that defines what information will be provided to the various entities that subscribe to the incident, both internally and externally.

As mentioned in previous sections, NG9-1-1 functional elements send EIDOs based on agency policies.

Recommendation on the primary EIDO will guide the PSAP to reflect on their relationships, legal agreements or set of policies determining the information carried by EIDOs exchanged between agencies.

EIDO transmissions follow a subscribe/notify model, where a notifier provides information about an incident to a subscriber.

A notifier executes policies that control what information is distributed to a subscriber.  These data sharing policies (“data policies”) are data documents that are based on PSAP operational processes and the requirements of each organization and any applicable jurisdictional laws.

The subscriber of information shall establish internal policies on how to subscribe and the types of conditions or status updates being requested from the various agencies or entities based on filters.

The PSAP Credentialing Agency (PCA) issues certificates to public safety agencies that have been vetted by the appropriate Registration Authority (RA).  This certificate allows the NG9-1-1 system to analyze the trustworthiness of agencies that send subscription requests to other agency incidents.  For example, a mutual aid partner that has been issued a certificate by the PCA may be provided more information than a certified agency that doesn’t have a mutual aid agreement.  In a situation where the IDX receives a subscription request from a non-certified agency then it will send the agency a subscription rejection if this is the response defined in the DRM policy. The process for certifying Non-Public Safety entities has been identified for future work.

## The Role of Data Rights Management Policy in the sending and receiving of incident data in an EIDO

In Legacy 911, telecommunicators relate incident information to agencies and individuals based on their roles, established information sharing policies, and applicable laws.  While it’s still the responsibility of the 911 Authority to establish data sharing policies in NG9-1-1, the IDX relates incident information automatically to subscribers based on how the 911 Authority has configured data rights in its vendor’s system(s).  The EIDO data sharing policies are the rules the software uses to make decisions regarding which subscribers are provided what data.

Such rules should be dependent on the level of trust the 911 Authority has in the subscriber as well as disclosure restrictions and the subscriber’s role(s).  Further considerations will be detailed in the following section.  See ‘Data Rights Management Diagram’ for an overview of how the level of trust a 911 Authority has in a subscriber should impact the amount of information they provide.

EIDOs communicate changes in the state of an emergency incident between emergency systems functional elements (FEs), public safety agencies and other entities. Agents and telecommunicators will never work directly with EIDOs.  They will take actions in the user interface that trigger EIDOs to be transmitted.  The information that is shared with a recipient will vary based on the configuration of the data rights management policy and what kind of request a recipient has made. A typical PSAP would also include subscriptions for key stakeholders such as law, fire and EMS.

The 911 Authority will be required to configure data rights in vendor systems both for internal use within the PSAP and for external public safety stakeholders.  While these may include neighboring PSAPs the authority has a mutual aid agreement with, these can also include PSAPs with which there are no formal agreements, including those outside the authority’s region, state or province.

When configuring data rights in Agency applications, per NENA-STA-024.1.1-2025 “The data rights management policy MUST conform to local, state/provincial, and federal laws and regulations.” Care should be taken to conform to laws related to criminal justice information and patient confidentiality like CJIS, CPIC and HIPAA such that a subscriber’s discipline and role within an organization dictate what information is available; for example, only providing personal health related information to EMS.

The 911 Authority will never modify the DRM policies associated with an EIDO it receives from another agency, nor will agencies receiving an EIDO sent by the 911 Authority be able to modify the DRM policy associated with it.  Their software should automatically be configured to respect the other agencies’ DRM policies by their vendor.  The following chart is an example of a matrix to assist in conceptualizing and managing data sharing rules.

- Matrix for Managing Data Sharing Rules

| Agency Type | PSAP (in area) Cooperating with Mutual Aid Agreements | PSAP (out of area) No Mutual Aid Agreement- Ex. Nearby – Same State/Province/County | PSAP (out of area) No Mutual Aid Agreement Ex. – Out of State/Province/County | Gov’t Services | Large Scale Disaster Recovery | Non-Public Safety | Media |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PSAP (in area) | Automated Policy (incident info yes/no) ☐Unrestricted ☐Confidential? ☐Fire? ☐Police? ☐EMS? | Automated Policy (incident info yes/no) ☐Unrestricted ☐Confidential? ☐Fire? ☐Police? ☐EMS? | Automated Policy (incident info yes/no) ☐Unrestricted ☐Confidential? ☐Fire? ☐Police? ☐EMS? | Future work | Future work | Future work | Future work |
| PSAP (out of area) | Automated Policy (incident info yes/no) ☐Unrestricted ☐Confidential? ☐Fire? ☐Police? ☐EMS? | Automated Policy (incident info yes/no) ☐Unrestricted ☐Confidential? ☐Fire? ☐Police? ☐EMS? | Automated Policy (incident info yes/no) ☐Unrestricted ☐Confidential? ☐Fire? ☐Police? ☐EMS? | Future work | Future work | Future work | Future work |
| Government Service | Future work | Future work | Future work | Future work | Future work | Future work | Future work |
| Large Scale Disaster Recovery | Future work | Future work | Future work | Future work | Future work | Future work | Future work |
| Non-Public Safety | Future work | Future work | Future work | Future work |  | Future work | Future work |
| Media | Future work | Future work | Future work | Future work |  | Future work | Future work |

Agents and telecommunicators will never work directly with EIDOs.  They will take actions in the user interface that trigger EIDOs to be transmitted.  The information that is shared with a recipient will vary based on the configuration of the data rights management policy and what kind of request a recipient has made.

Policy Decision Considerations:

-	is this a request for awareness or BOLO memo scenario? For specific incident use request for service in conveyance. Maybe requests go through standard mechanisms and EIDO’s follow per standard and policy store.

## Security of Incident Data

The incident information shared via the IDX and other functional elements will adapt dynamically based on the subscribing agency's role and changes in incident status. Within the PSAP environment, Federal, State, Province, Territory and local government laws and regulatory requirements should govern sensitive data transmission across fire, EMS, and law enforcement. Public safety stakeholders will require access to less redacted data, while agencies outside of public safety might receive only targeted information relevant to their specific resource request(s).

Inter-PSAP communication for mutual aid shall prioritize essential data points such as incident location, nature, and required resources. withholding additional incident data until the resource request is accepted by the receiving agency For agencies with frequent collaboration, real-time updates on resource locations and availability will be crucial. Sharing of incident data shall seamlessly integrate with existing provisioning processes and communication workflows within each PSAP. Maintaining a common format for incident data streamlines dispatcher workload and facilitates a common operating picture between PSAPs and responders.

A typical PSAP should include subscriptions for key stakeholders: law enforcement, fire, EMS, towing services, utilities, and highway departments. While emergency service agencies will receive incident information based on the Data Rights Management policies established by their PSAP with variations based on their roles, support agencies like utilities and highway departments will primarily receive location and incident details essential to their response.

Subscriptions for emergency services shall employ an automated flow process driven by CAD recommendations. Dispatchers can readily add frequently used support agencies as resources within the CAD system.

Subscription requests should be streamlined through pre-configured workflows and initiated during mutual aid requests or incident transfers. Crucially, both requesting and accepting agencies must adhere to their respective policies and all applicable laws. The Law Enforcement Data Exchange, for example, must comply with Canada’s regulatory framework related to CPIC and state\territory regulations, while medical information sharing in the United States will strictly adhere to HIPAA privacy and security rules.

To safeguard sensitive data, subscription requests shall follow a phased approach. Initial requests for mutual aid will provide only essential information (nature, location, resources). If the receiving PSAP cannot fulfill the request, sensitive data remains protected. Upon acceptance, incident data flows freely for the incident's duration, tailored to each agency's function.

# (Impacts and considerations)

NOTE to document editor: This section is optional. If this section is not needed, delete the section title and the text.

Use this section to convey any additional information that the Working Group believes is important for the reader to understand. Examples of impacts and considerations include the following:

- operational

- technical

- security-related

- level of effort

- time required to implement

- cost or cost-recovery

- any other information the group believes the reader should understand when using this document

The organization of this section and any subsections is at the discretion of the editor and the Working Group.

### Policy Considerations for EIDO Sharing

Local agencies SHALL establish clear policies governing access to incident information based on agency role and data types. For example, a policy might stipulate that all law enforcement agencies responding to an incident receive comprehensive incident data until released by the incident commander. However, to protect potential investigations, policies should restrict access for other disciplines. This should involve limiting data visibility within 911 systems and utilizing encrypted communication channels for law enforcement.

To place the above matrix in context, consider that all the data components of an EIDO listed in 1.4 MUST have DRM policies defined in your NG9-1-1 vendor system(s) down to the level of each individual element of each data component.  The system will automatically send incident updates to subscribers based on these policies.  As an example, consider the Notes Data Component which may contain notes taken by emergency service agents and responders on scene at an incident.  EIDO’s containing this data component could easily contain information which, by law, has associated disclosure limitations.  How your PSAP defines DRM policies in your vendor system(s) defines who receives what information in an EIDO.

Some basic questions you might ask yourself when defining your DRM policies may include… Is the information sensitive?  Do you have a mutual aid agreement with the subscribing agency?  What’s the subscriber’s role (Law, Fire, EMS, PSAP)?  How far away are they from your agency?  How much do you trust the subscriber? The amount of information you’re willing to provide a mutual aid partner may vary significantly from what you’re willing to provide to an agency two hundred miles away but you provide the more distant subscriber with ‘something’ because they’re trusted by the PCA.  The matrix is a guideline for general considerations you should take into account when defining your DRM policies.

That said, traditional agency roles may vary for a limited time in an incident response.  For example, first on-scene may be law enforcement, and the situation requires an officer to begin initiating first aid. In such a situation the responding officer may have a need-to-know health related information specific to an individual.  If the 911 system is preconfigured to only release this kind of health-related information to EMS agencies, then the telecommunicator may need to convey this information to the responding officer orally.  Alternatively, the PSAP may choose to establish policies to deal with situations where preconfigured Data Rights Management solutions are inadequate.  Such consideration may be necessary because as previously mentioned, in the NG9-1-1 environment vendor systems SHALL be configured to automatically send incident data based on your agency’s DRM policy.  The system won’t know when to make exceptions.

The Data Rights Management Diagram below helps to summarize the general data sharing levels a PSAP might have based on the role and level of trust the PSAP has in the subscriber.

NG9-1-1 systems necessitate granular policies that define data access for each subscriber role and the level of trust the policyholder has in the subscribing agency. These policies should categorize the data components outlined in NENA-STA-021 down to the element level data types (e.g., medical, warrant, scene safety) and incorporate all relevant legal and security regulations. By automating "if-then" decision-making processes, these policies alleviate the manual burden currently placed on dispatchers.

PSAPs already have established contingency policies for situations during system overflows and tied up resources. These plans provide a roadmap to defining what data is exchanged during emergencies. So, by using your existing relationships as a framework, develop your data sharing policies in a similar way being cognizant of data privacy and stewardship. Assuming you have a good relationship with at least one other PSAP leverage that asset so that you both understand how you will subscribe to and exchange data adhering to polices you have created.

This shift towards automated incident data transmission policies necessitates a heightened focus on security. Robust s Security measures must be implemented to protect sensitive information transmitted and received via EIDO throughout the incident lifecycle. At the end of the day, you are as good as your Data Rights Management policy.  NENA suggests working with existing support agencies in your jurisdiction such as your NG9-1-1 system vendor(s), your Information Technology Department (or equivalent), Human Resources and legal team to see if they’ve already established a data security policy for your jurisdiction.  If existing policies generally conform to the NENA Security for Next Generation 9-1-1 Standard (NG-SEC) (NENA-STA-040.2-2024) then work within your existing framework to establish your EIDO DRM policies.

If your jurisdiction doesn’t already have a DRM policy in place, then use section 4 of NENA-STA-040.2-2024 as a baseline for a minimum DRM security policy.  If you’ve already established a DRM policy with mutual aid partners in accordance with the NG-SEC standard, you can use this as a starting point for a larger policy for both internal and external agencies and roles.

### Transitioning from Human-Driven to Automated Policies

While many automated data sharing policies should mirror existing human-driven practices, those subject to frequent change (due to staffing fluctuations, social factors, etc.) may need toremain under human control. In some situations, vendors may need to create systems in such a way that 911 administration can effectively ‘disable’ the automatic

transmission of incident data.  For example, if a high-ranking government official is involved in a vehicle crash, a 911 admin may wish to highly restrict information related to that incident. However, by transitioning stable, rule-based policies into automated workflows, PSAPs can significantly enhance efficiency and reduce the cognitive load on dispatchers. There is also the issue of exposure to information and imagery that some find disturbing especially with the nature of this work. It is recommended that guidelines from the above standard are implemented as soon as practicable.

### Transition to Security Policies

This shift towards automated data policies necessitates a heightened focus on security. Robust security measures must be implemented to protect sensitive information throughout the incident lifecycle. At the end of the day, you are as good as your Data Rights Management policy. We suggest reading NENA Security for Next Generation 9-1-1 Standard (NG-SEC) specifically section 4 which deals with policy management. This gives good guidance on implementing security policy from the management perspective. Below are some specific security considerations to address the aforementioned.

Historically, decisions about who could see incident information lived outside the 9-1-1 system, i.e., in memoranda of understanding, mutual-aid agreements, and local standard operating procedures, with telecommunicators and supervisors applying those rules manually, one incident and/or request at a time. In the NG9-1-1/i3 environment, that same intent is expressed digitally as Data Rights Management (DRM): element-level rules, defined per subscriber role and level of trust, that the vendor system enforces automatically and that are governed through the PCA subscription model. The substance of your existing agreements doesn't disappear, it is translated from prose and handshake into machine-enforceable policy.

The DRM policy itself - translating your existing rules into XACML(eXtensible Access Control Markup Language) a standard that provides a policy-based framework for defining and enforcing authorization rules, determining who can access specific resources and under what conditions.

In the legacy environment, your data-sharing rules lived in people's heads, in MOUs, and in SOPs, and a telecommunicator applied judgment at the moment of release. In i3, those rules are expressed in XACML v2.0, the OASIS standard that NENA i3 (NENA-STA-010.3, §5.6) adopts as the mechanism for DRM and that the NGCS and PSAP systems enforce automatically. You will not normally hand-write XACML, your vendor's policy administration tooling generates it and holds it in the Policy Store. However, you are responsible for getting the logic right and stating it precisely enough that a machine can apply it with no person in the loop.

The most important mental shift is to default-deny. A person could improvise and share something reasonable in a situation the rules never anticipated; XACML does the opposite, anything you do not explicitly permit is withheld. So the work is not to write down what you want to protect, it is to enumerate exactly what you are willing to expose, to whom, and under what circumstances. Everything else is denied by default.

To get there, take each of your existing policies, your MOUs, mutual-aid understandings, and local release practices, and decompose them into the four questions XACML asks of every request:

·         Who is asking (Subject). The subscribing entity, identified by its PCA-traceable credentials: agency role (Law, Fire, EMS, PSAP), the specific agency, mutual-aid status, and the level of trust you assign it. This is where the role-and-trust groupings from the matrix in the previous section become concrete, enforceable attributes.

·         What data (Resource). The specific EIDO data components and, critically, the individual elements within them, classified per NENA-STA-021, location, incident type, person/medical, vehicle, warrant, scene-safety, and so on. "Share the incident" is not specific enough; the policy must say which elements.

·         What operation (Action). Typically, to receive or subscribe to that element as the incident updates.

·         Under what circumstances (Environment / Conditions). Context such as incident type, whether the incident commander has released information, the subscriber's relationship or distance, and whether the subscription is scoped to a single incident ID or is a standing qualified-filter subscription.

Each rule then resolves to a Permit or Deny, optionally carrying Obligations, actions the system must take when it enforces the decision, such as redacting a field, logging the disclosure, or flagging that the information may need to be conveyed by other means. Obligations are how you carry over the "yes, but…" clauses of your existing policy.

The practical method, and the deliverable your vendor and developers will need from you is to write each rule in plain language first, validate it against your sharing matrix, and only then have it encoded in the Policy Store. For example: "Permit EMS-role subscribers dispatched to this incident to receive medical elements; deny medical elements to all other roles." Working in plain English keeps the policy auditable by the people who actually own the data, and it surfaces conflicts before they are buried in XML.

Two issues deserve deliberate attention as the set grows. First, conflicts: when more than one rule could apply to the same request, XACML resolves them with a combining algorithm, and for life-safety data a deny-overrides posture, any applicable deny wins and is the conservative choice. Decide this on purpose rather than inheriting a vendor default. Second, exceptions: as noted earlier, the system cannot improvise — it releases exactly what the policy says and nothing more. Encode the foreseeable exceptions as environment conditions (for instance, an incident-commander release flag) and pair the automated policy with a documented manual path for the situations no rule anticipated.

Finally, treat the policy set as living. Validate it against representative scenarios before go-live to confirm it exposes only what you intend, review your role-based access at least annually as NG-SEC requires, and revise the rules as roles, partners, and statutes change.

The security of the EIDO as it moves

Confidentiality, integrity, and authenticated transport, addressed by the Conveyance standard (NENA-STA-024) and i3 transport protections such as TLS using PCA-traceable credentials.

Your organization's overarching security governance

Acceptable use, access control, data classification, incident response, and the rest of the documented program a PSAP maintains. This is the domain of the NENA Security for Next Generation 9-1-1 Standard (NG-SEC, NENA-STA-040.2-2024). NG-SEC describes how to stand up and run that program from a management perspective; it is the enterprise context your DRM policies live inside, not a substitute for them.

## Security of EIDO Data

EIDO data encompasses a wide range of information, from statuses of resources, Personally Identifiable Information (PII), HIPAA protected information, to sensitive information respective to an open investigation. With advances in technology all data is susceptible to the growing capabilities of eavesdroppers and attackers. The Next Generation (NG9-1-1) systems, applications, and flow process security must develop best practices for the safety of both the data and ultimately the responders and greater community. NENA-STA-024.1.1-2025, Conveyance of Emergency Incident Data Objects (EIDOs) between Next Generation (NG9-1-1) Systems and Applications addresses the security for the EIDO during its life within technology components. Remaining security risks that must be addressed lie within the human factor.  And whereas, in previous 9-1-1 operations, the risk was always present, even exacerbated by growing staffing turnover rates. In NG9-1-1 those risks can be mitigated to a large degree through the subscription policies addressing Data Rights Management, developed by the PCA. Today agencies are cautious of data risks to the point of hindering interoperability. As subscriber policies are established for the protection of data, interoperability and regional resource allocation could become a commonality.

We suggest reading NENA Security for Next Generation 9-1-1 Standard (NG-SEC) specifically section 4 which deals with policy management. This gives good guidance on implementing security policy from the management perspective. Below are some specific security considerations to address the aforementioned.

Remaining security risks that must be addressed lie within the human factor.  In Legacy 9-1-1 operations, the risk was always present and is exacerbated by growing staff turnover rates.  In NG9-1-1 these risks can be mitigated to a large degree through the policies addressing Data Rights Management which are implemented by the agency.  An agency can create groupings, such as those detailed in the matrix in the previous section, with predefined data rights based on role(s).

NENA-STA-024.1a-2023 (2.10 Security) “All transports specified in this document are protected by TLS using credential (for those entities within an ESInet) traceable to the PSAP Credentialing Agency (PCA)”.  We are saving government services, non-public safety entities and media for future work so detailing security for these entities may be out of scope for this version of the document.For the security of EIDO creation reference the Conveyance and JSON Documents as appropriate.

For Best Practices for EIDO’s we want to apply these criteria (and then see if Conveyance can verify?)

Agency Roles

Agent Roles

## What best practices should be applied to EIDO Management

Recommendations based on anticipated issues and concerns:

Make sure you can transfer to agencies you know nothing about and receive from those you know nothing about as well.

You would want a policy for every legal agreement you have or one policy that covers them all. (agency policy vs software policy).

Subscriptions – software approved vs. human approved.  When it’s an agency you are not familiar with, you will want to know who is subscribed.  Who wants to know and why?

Include definitions and what goes where….how data rights management fits into the policy store, policy routing.

Default – to reject a subscription because it’s not a call they transferred to you.

Definition of legitimate will vary.

Querying loggers use case

1.	NG911 functional elements

2.	Public safety applications within the PSAP

3.	Originating PSAPs and assisting PSAPs

4.	Emergency responder applications

5.	Other law enforcement, fire, and EMS entities

Incorporate some of this language into the best practices section as appropriate:

Small PSAPs may not have much IT support. As moving to NG9-1-1, make sure you have IT support to make the transition correctly. You must prepare or you’ll have multiple kinds of new software systems, you need an appropriate network, connection to neighboring agencies, etc. Have conversations with them in the preparation phase. Don’t be afraid to have a meeting with all your vendors in one room – it helps to make sure everyone is on the same page, and to prepare for the ultimate integrations. 
Then look at your policies and procedures. You have procedures to do all of this manually. What data is shared with other entities? Translate to data sharing policies. Write it all out. RACI = Responsibility, Accountability, Consult, Inform Matrix. Make decision trees and flow charts to make sure you and your partners and vendors understand what you’re doing and how you plan to get to your goal(s). Other methods include using sticky notes with lines or strings showing the interdependencies. All of this sets the stage for positive change management, which will help you all make a smooth transition.

Start slow and build on that when creating your management policies. Focus on what you’re doing today and figure out how to replicate and automate that.

Dan talks about rules for sharing data – you own your data. You can’t share data you got from another agency, but you can share their link to the data and a third party can request the data via the link – the owner decides what to give the third party. The PCA certificates ensure that each party knows who the other party is and the data owner can use its own policy rules to decide what it will share with the requesting party.

As EIDO is not currently widely available in production environments, we believe best practices will continue to emerge with future deployments.

It’s important to understand that with the adoption of EIDOs as the standard format for incident data transmission and receipt that your data sharing relationship with other agencies is no longer limited to those you have a mutual aid agreement with.  Interagency agreements are not necessary to exchange EIDOs between functional elements of different agencies.  Each agency must have rules in their Data Rights Management (DRM) policy to cover exchanges between agencies, including agencies unknown at policy writing time.  Such agencies may be outside your region, state/province or country.

There is no requirement to provide an unfamiliar subscriber with the same amount of incident data you would provide a known public safety partner, but they are trusted by the PCA, so you provide them with something regardless of whether you know them or not.

If you receive a subscription request from an entity that hasn’t been issued credentials by the PCA you have no obligation to provide them with anything.  If your policy is to reject their subscription request, then be sure to document this to your policy and verify your systems are configured to reflect this.

## Inter-Agency Agreements

Traditionally Interagency agreements, i.e. Mutual Aid Agreements, have been between entities with an established relationship.  The established relationship provided both entities were aware of the available equipment and capabilities.  With these arrangements details such as reimbursement, insurance, method of contact and time required to respond were agreed upon when agreements were signed.  With the implementation of the EIDO the traditional interagency agreements may continue to be implemented between entities for established relationships; with additional agreements needing to be considered for the new scope of resources and capabilities entities may be aware of through EIDO implementation.

Agencies may need to refine their agreements to cover which EIDO data fields they will share with a particular agency. Agreements will continue, but will migrate from existing communication models to the EIDO models. Since NG911 will include non-traditional CAD to CAD partners, Agencies will need defined policies of which data to share those partners.

Interagency agreements are not necessary to exchange EIDOs between functional elements of different agencies. Each agency must have rules in the data sharing policy to cover exchanges between agencies, including agencies not known at policy writing time.

# (recommendations for additional development work)

NOTE to document editor: This section is optional. If this section is not needed, delete the section title and the text.

Recommendations for additional development work generally include information that should be considered in a future version of the document or is intended to be referred to another NDG Committee, Working Group, or Standards Development Organization. In general, additional development work can be considered as a list of important topics that the Working Group recommends for further development work.

The following list is intended to help the Working Group decide what to include as additional development work.

- Information that is valuable to the topic, although deemed out-of-scope for the current version’s Charter. These items can be considered in a future version of the document, and many times will support the planning of the document’s next version.

- Information or a topic that the Working Group recommends be considered by a different NDG Committee or Working Group, or Standards Development Organization (i.e., IETF, ATIS, APCO). If known, include the NDG Committee or Working Group, or the Standards Development Organization in the item description.

- Any information added to this section should be as complete as possible due to the future nature of the efforts. The current Working Group should not assume that it will be the Working Group assigned to support the future work.

Additional development work, whether as a revision to the document or a referral, is supported by the Issue Submission Form (ISF) process for consideration of the NDG membership and direction by the DSC. More information on the ISF process can be found by visiting https://www.nena.org/page/Submit_IssueNDG.

Upon publication, the Working Group Co-Chairs will review this section with their Committee Co-Chairs to determine the next step for each item listed in this section. This would include sending recommendations to external organizations or generating ISFs for NDG consideration.

This document references existing NENA [Standards (STA), Requirements (REQ), Information (INF), Reference (REF)]. Additional work may be required as noted below.

NOTE to document editor: The section can be structured with numbered section headings, or the table below can be used to document future work. Choose one format and remove the other from this section.

Suggested Heading format:

## [Future Work Item] Push

NOTE to document editor: Provide a distinct name for the future work item.

New Input:  Details will be left to future versions of the document.

Version 1 will only have subscriptions. Areas identified for future work:

Are we entitled to recommend what should be the primary EIDO information for each type of protocols?

(Push)

·     Request for Service

·     Request for Resource

·     Request for Awareness

Pushes (EIDO are created when certain criteria is met, example triggers)

Conveyances (Request for Service, Request for Resources and Request for Awareness are all forms of Conveyance which relay a function needed by an Agency)

Document what we do today – how do people know what they can/can’t share over the phone

Define the “If this, then that” statements (IFTTT).  Start with Mutual Aid Agreements.  Ops folks bring an example to next meeting.

Future work as related to EDXL-RM, in development in the conveyance group

## [Description] What can be sent?

NOTE to document editor: Describe the future work item, its background, a justification for why it should be done, and if applicable, the document section number.

- Requests for service

  - Call information on calls disconnected prior to transfer

  - Assistance for response from neighboring agency

  - Death notifications

  - Panic alarms

  - Aircraft, boat, or train incident

- Requests for resources

  - Bomb squad

  - K9

  - Ambulance

  - Law enforcement response

  - Haz-Mat

  - Power company/gas company

  - Internal call outs

- Requests for awareness

  - BOLOs

  - ATLs

  - Emergency Alerts to hospitals

  - Hospital notifications of active diversion

  - Road closure information

  - Power outage and gas leak information

  - Extreme weather information, i.e. flooded areas

  - Power plant emergency alerts

  - FEMA/Emergency Management alerts

  - Video, photos, and other media

    - Driver’s license photos

    - Social media photos

- When utilizing EIDO’s you must understand the roles and Information relayed by not only the EIDO but also the conveyance protocol used.  The three types of protocols are Request for Service, Request for Resource and Request for Awareness. The EIDO itself will provide call and/or incident information but the protocol by which the EIDO was delivered will provide incident information also.  The reason you must consider the difference is that when you are deciding on what information will be in an EIDO you must understand what effect the conveyance of the EIDO will have.  For example, if the EIDO uses the conveyance protocol of Request for Service, the location should be included in the EIDO body.  If the conveyance protocol of Request for Resource is used, the EIDO should include the type of incident.  An EIDO using a Request for Awareness may not require an ETA of responding units or apparatus.

- Another major difference between EIDO Data fields and Conveyance protocol is the acknowledge and response required by protocols. These responses convey more than just machine language of passing messages.  By acknowledgement of message being received, the PSAP doesn’t have to wonder or anticipate whether the information was relayed. If the EIDO was conveyed and acknowledged but a response not received then depending on the urgency of the Request, the PSAP may need to make another Request of another ECC.  If an acknowledgement was not delivered back, then the ECC knows to try and make human contact or move on.

### [Recommended Assignment]

NOTE to document editor: Include a recommended assignment such as a future version of this document, NDG Committee, Working Group, a new Working Group, or external Standards Development Organization (i.e., IETF, ATIS, APCO). If unknown, work with the Committee Co-Chairs to identify an appropriate assignment.

Suggested Table format:

Table 4-1 Future Work for Consideration

| Recommended Assignment | Future Work |
| --- | --- |
| [Recommended Assignment] | [Future Work Item] [Description of future work item, with section number if applicable] |
| [Recommended Assignment] | [Future Work Item] [Description of future work item, with section number if applicable] |

# Abbreviations, Terms, and Definitions

Use this table to identify all terms that are unique to NENA or the 9-1-1 system or are important to this document. All abbreviations used in this document are to be included in this table. If only used once within the document, consider fully spelling out the term without its abbreviation so it does not need to be added to this table. Creating abbreviations that are not subsequently used in the document is not recommended.

Compare your definitions with the current NENAkb Glossary to avoid unnecessarily creating conflicting descriptions of the same term. Review terms just before each approval ballot since the NENAkb Glossary is continually updated and definitions may have changed.

Please add terms and abbreviations to the table using the following format:

- Place term or abbreviation alphabetically in the Term or Abbreviation (Expansion) column. For abbreviations, add the expansion (the spelled-out words) in parentheses, e.g., FCC (Federal Communications Commission).

- Provide a brief definition or description for the abbreviation or term.

- In the Definition column, you may also identify other Relevant NENA Documents or External References that provide greater detail about the term or abbreviation. URLs added to cited references will not be included in the NENAkb Glossary.

Instructions for Recommending Changes to the NENAkb (NENA Knowledge Base) Glossary

Use the third column to recommend changes to the NENAkb Glossary or confirm acceptance of the Glossary’s definition in this document. The CRM team will remove the third column before the approved document is published. These options are available in the third column.

Select Recommendation drop-down box:

- In Glossary—The term and the definition in the document are the same as in the NENAkb Glossary. Please use the definition in the NENAkb Glossary.

- Add—The term does not exist in the NENAkb Glossary and meets the criteria for inclusion. You may also identify other Relevant NENA Documents that provide greater detail about the term or abbreviation.

- Delete—The term exists in the NENAkb Glossary but is no longer needed or does not meet the criteria for inclusion.

- Don’t Add—The term is not in the NENAkb Glossary and does not meet the criteria for inclusion OR the WG needs a unique definition in this document other than what is in the NENAkb Glossary.

- Modify—Update the NENAkb Glossary with this term and definition. If the term originated in a different NENA document, consult with that document’s Working Group (or its Committee if not currently active) to develop a single, agreed-upon definition.

- Other—Use this option to suggest other actions such as adding this term as a “Related Term” or “Also Known As” term to the definition of another term in the Glossary.

- Insert Explanation—If “Other” is selected OR a definition was modified in collaboration with another WG, OR any other explanation is needed, enter the details in the Insert Explanation text box (3rd column). Or else, delete the Insert Explanation text box.

See the NENA Knowledge Base (NENAkb) Glossary [1] for a list of terms and abbreviations used in NENA documents. Abbreviations and terms used in this document are listed below with their definitions as they apply to this version of the document. Please be aware that terms and definitions in the NENAkb Glossary may be different than what is shown in this document.

| Term or Abbreviation (Expansion) | Definition | Recommendation for NENAkb Glossary |
| --- | --- | --- |
|  |  | Select Recommendation Insert Explanation |
| LS (Location Server) | General term for the entity responsible for obtaining the location of the User Equipment (UE). See 3GPP TS 23.167 | Add Add as a related term under IMS. |
| NGCS (Next Generation 9-1-1 Core Services) | The set of services needed to process a 9-1-1 call on an ESInet. It includes, but is not limited to, the ESRP, ECRF, LVF, BCF, Bridge, Policy Store, Logging Services, and typical IP services such as DNS and DHCP. The term NG9-1-1 Core Services includes the services and not the network on which they operate. See Emergency Services IP Network. | Modify Definition modified in collaboration with the i3 Architecture WG |
| SHAKEN (Signature-based Handling of Asserted Information Using toKENs) | An industry framework for managing the deployment of Secure Telephone Identity (STI) technologies with the purpose of providing end-to-end cryptographic authentication and verification of the telephone identity and other information in an Internet Protocol (IP)-based service provider voice network. See ATIS-1000074-E | Add |

# References

NOTE to document editor: Remove reference [4], [5], and [6] if the Describing Artifacts section was removed from the beginning of the document and it is not referenced elsewhere in the document.

- National Emergency Number Association. “NENA Knowledge Base Glossary.” Updated [insert the date indicated at “Approved updates to NENA Knowledge Base Glossary last made” as found at the cited link in the format Month DD, YYYY]. https://kb.nena.org/wiki/Category:Glossary.

- Internet Engineering Task Force. Key words for use in RFCs to Indicate Requirement Levels. S. Bradner. RFC 2119, March 1997.

- Internet Engineering Task Force. Ambiguity of Uppercase vs Lowercase in RFC 2119 Key Words. B. Leiba. RFC 8174, May 2017.

- National Emergency Number Association. NENA Rules for Use of Version Control Repositories. NENA-ADM-012.1-2022. Alexandria, VA: NENA, approved November 8, 2022.

- OpenAPI Initiative. OpenAPI Specification 3.1.0 Released. https://www.openapis.org/blog/2021/02/18/openapi-specification-3-1-released, February 18, 2021.

- World Wide Web Consortium (W3C). W3C XML Schema Definition Language (XSD) 1.1 Part 1: Structures. S. Gao, C. M. Sperberg-McQueen, and H. Thompson. https://www.w3.org/TR/xmlschema11-1/, April 5, 2012.

NOTE to document editor: All sources used in the development of the document must be cited within the body text of the document. The reference citation is constructed in accordance with the Chicago Manual of Style (CMOS) (examples are provided below).

When citing a NENA document, do not include an errata version letter in the document number and use the NENA Standards page URL (https://www.nena.org/page/standards) as the link to the document. Links to material outside of this document should be limited to the web page where the referenced document may be downloaded, as opposed to a link directly to the specific document. Questions regarding formatting references should be directed to the Committee Resource Management team at crm@nena.org.

- [National Emergency Number Association. NENA i3 Standard for Next Generation 9-1-1. NENA-STA-010.3-2021. Alexandria, VA: NENA, approved July 12, 2021.]

- [National Emergency Number Association. Emergency Incident Data Object (EIDO). NENA-STA-021.1.1-202Y. Alexandria, VA: NENA (forthcoming).]

- [Internet Engineering Task Force. Session Initiation Protocol. J. Rosenberg, H. Schulzrinne, G. Camarillo, A. Johnston, J. Peterson, R. Sparks, M. Handley, and E. Schooler. RFC 3261, June 2002.]

- [Association of Public-Safety Communications Officials International. APCO International Core Competencies and Minimum Training Standards for Public Safety Communications Quality Assurance Evaluators (QAE). APCO ANS 3.106.2-2017. Daytona Beach, FL: APCO, approved September 12, 2017.]

- [International Telecommunications Union. The international public telecommunication numbering plan. Recommendation E.164 (11/10), November 18, 2010.]

- [National Institute of Standards and Technology. “Cybersecurity Framework.” Accessed December 6, 2024. https://www.nist.gov/cyberframework.]

- [Trafficking Victims Protection Act of 2017, 34 U.S.C. § 10101 (2018), https://www.congress.gov/115/plaws/publ393/PLAW-115publ393.pdf.]

- [U.S. Department of Justice, Federal Bureau of Investigation. Criminal Justice Information Services (CJIS) Security Policy. CJISD-ITS-DOC-08140-5.8. Washington, DC: USDOJ, June 1, 2019.]

- World Wide Web Consortium (W3C). W3C XML Schema Definition Language (XSD) 1.1 Part 1: Structures. S. Gao, C. M. Sperberg-McQueen, and H. Thompson. https://www.w3.org/TR/xmlschema11-1/, April 5, 2012.

NOTE to document editor: Within the body text of the document, a numerical reference to the citation is provided at the end of the applicable text using the Word cross-reference function. The first occurrence of a cited work shall list the italicized name, followed by its document number (if provided) without a version number or errata letter. For example: NENA i3 Standard for Next Generation 9-1-1, NENA-STA-010 [5]. Subsequent citations of this document would be formatted using one of the following options:

- NENA i3 Standard for Next Generation 9-1-1 [5]

- NENA-STA-010 [5]

For References cited within the body text of the document and where it is important that the reader only go to the “specific version” of the targeted document, the Reference should include the targeted document’s version number.

It is not advisable to include specific locations “within” a targeted document, such as a section number, paragraph number, or line number. Those specific locations are more likely to change when the targeted document is updated, even if the text referred to did not change.

# Appendices

## Appendix X—[Appendix Title]

NOTE to document editor: The Appendices section is an optional section intended to include additional information referenced in your document. If this section is not needed, delete the section title and the text.

Each Appendix should begin on a new page. Name each Appendix as Appendix A, Appendix B, etc.

ACKNOWLEDGMENTS

The National Emergency Number Association (NENA) [insert the applicable Committee, Subcommittee, and/or WG names] [developed, revised, reaffirmed] this document.

NENA recognizes the following industry experts and their affiliated organization for their contributions to the [development, revision, reaffirmation] of this document.

NOTE to document editor: The table below shall contain the names of the persons who have actively contributed (verbally or in writing) to the development of the document as determined by the Working Group Co-Chairs. The employer organization shall be the name of the person’s affiliated organization, or “Individual” if not affiliated with an organization, at the time the document is approved by the NENA Board of Directors. Where appropriate, include a member’s ENP or GISP status. Members should review their entry in this table before publication.

| Members | Organization |
| --- | --- |
| Committee Co-Chair Name, XXX Committee Co-Chair |  |
| Committee Co-Chair Name, XXX Committee Co-Chair |  |
| WG Leader Name, Working Group Co-Chair |  |
| WG Leader Name, Working Group Co-Chair |  |
| WG Member Name, Technical Editor [if different than WG Leader] |  |
| WG Member Name, Note Taker [if different than WG Leader] |  |
| WG Member Name |  |
| WG Member Name |  |
| WG Member Name |  |
| WG Member Name |  |
| WG Member Name |  |

Special Acknowledgments

NOTE to document editor: If a staff member is assigned to a specific working group for administrative support, the CRM team will adjust the text as needed. If there are questions about use of the Additional names and description of reasons for acknowledgement line, please contact the CRM team.

NENA’s Committee Resource Management Team has facilitated the production of this document through the prescribed approval process.

[Additional names and description of the reasons for acknowledgment]

The [Working Group Name] Working Group is part of the NENA Development Group that is led by:

Wendi Rooney, ENP, and Lisa Dodson, ENP, Development Steering Council Co-Chairs

Brandon Abley, ENP, Chief Technology Officer

April Heinze, ENP, VP, Chief of 9-1-1 Operations

GENERAL DOCUMENT EDITING HELP

NOTE to document editor: NENA documents follow the Chicago Manual of Style (CMOS). Document editors are to use the following guidelines when editing NENA documents. If there are any questions regarding this section, contact the CRM team.

Delete this entire section from the document before submitting the document for external review.

Navigating the Styles Gallery and Styles Pane

NOTE: Document editors MUST use the styles listed in the Styles Gallery in this template. These predefined Word styles are set to follow NENA’s required font and formatting options.

The most frequently used styles are shown in the Styles Gallery on the Home tab.

Open the Styles Panel to see additional styles available in the document by clicking on the small arrow in the bottom right corner of the Styles Gallery.

To apply a style, select the text to be formatted, and then click on the desired style in the Style Gallery or Styles Panel.

Use the Font options on the Home tab to apply formatting (e.g., bold, italics).

Watermark

All pages must include a DRAFT watermark.

Font

Verdana 10 pt

When copying in text from a document, email, chat box, or other source, use the Keep Text Only paste option so text is added with the correct font and formatting.

Paragraph Formatting

Use the Word Style Body Text for text in the document’s main body. This adds visual space between paragraphs, eliminating the need for a blank line.

Messages/Code in Text

Font: Lucida Console 10pt

Left indent: .5

XML elements: shown in brackets (<...>)

JSON objects: shown in double quotes with brackets ({"..."})

Values: shown in double quotes ("...")

Examples:

<note>

<to>Dern Handy</to>

<from>Brandon</from>

<heading>Reminder</heading>

<body>Don't forget to book!</body>

</note>

{

"iFoo":12,

"sBar":"blurdybloop"

}

“urn:service:sos”

Footnotes

Cited documents, websites, and other references MUST be cited in the References section, not in footnotes. If something is important enough to include as a footnote, it is strongly recommended that the information be included in the document text.

Numbers: Superscript Verdana 7pt

Text: Verdana 8pt

Punctuation

A comma is placed before a conjunction (e.g., and, or) in a series of three or more.

Periods are followed by a single space.

Periods and commas are placed within closing quotation marks.

  - Exception: If file names, words, or other strings to be typed are within quotation marks, any punctuation that belongs to the surrounding sentence should appear outside the quotation marks.

Colons, semicolons, question marks, and exclamation points are placed outside closing quotation marks (unless they belong within the quoted matter).

CMOS prefers square brackets [ ] within parentheses instead of parentheses within parentheses.

The abbreviations i.e. (“that is”) and e.g. (“for example”) are placed within parentheses, where they are followed by a comma.

CMOS prefers to limit the abbreviation etc. (“and other things”) to parentheses. If used in a sentence, etc. is preceded by a comma. Do NOT use etc. at the end of a list that begins with for example, such as, e.g., and the like.

Non-breaking Hyphens

The CRM Team will insert non-breaking hyphens for 9-1-1 and all hyphenated words at time of publication. Editors should use regular hyphens during document development to avoid copy/paste of non-breaking hyphens being changed to spaces (no hyphen).

alt+shift+hyphen inserts a non-breaking hyphen which is treated as a letter, keeping the word on a single line.

Vertical Lists—bulleted vs. numbered

- Numbered lists are used to indicate the order in which tasks should be done, to suggest chronology or relative importance among the items, or to facilitate text references.

- A vertical list is best introduced by a grammatically complete sentence, followed by a colon.

- All items should be styled with the same syntax, either sentences or sentence fragments.

- All items should use the same verb tense.

- For bulleted lists, unless the items themselves form complete sentences, use lowercase for the first letter of each item (unless it is a proper noun) and omit periods.

- For bulleted lists whose items require more prominence, capitalization may be preferred; choose one approach and follow it consistently.

- Capitalize items in a numbered list even if the items do not consist of complete sentences. Closing punctuation is used only if items consist of complete sentences.

- If the items in a vertical list complete a sentence that began in the introductory text, commas or semicolons may be used between the items (use semicolons if the items include internal punctuation), a period should follow the final item, and no colon is used in the introductory sentence. A conjunction (e.g., and, or) before the final item is optional.

- Items in a vertical list should be short and to the point.

Vertical List Symbology and Indentation

- Hanging Indents: All use .25”

- Paragraph Indents and Spacing:

  - Don’t add space between paragraphs of the same style (check box)

- Paragraph Line and Page Breaks:

  - Widow/Orphan Control (keep box checked) *Note: This only affects a bulleted paragraph consisting of 2-3 lines. Bullets with a single line are not affected.

  - Keep with next (uncheck box)

Bulleted and numbered lists can be formatted by using the Format Painter tool to apply the formatting below to the text in the core document. They can also be created by using the styles listed in the Styles gallery on the Home tab or by using the Bullets and Numbering formatting options within the Paragraph group on the Home tab.

Bulleted List

- Left Indent .25

- Left Indent .25

  - Left Indent .75

  - Left Indent .75

    - Left Indent 1.25

    - Left Indent 1.25

      - Left Indent 1.75

      - Left Indent 1.75

Numbered List

- Left Indent .25

- Left Indent .25

  - Left Indent .75

  - Left Indent .75

    - Left Indent 1.25

    - Left Indent 1.25

      - Left Indent 1.75

      - Left Indent 1.75

Alpha List

- Left Indent .25

- Left Indent .25 (AA-ZZ: Hanging indent .38”)

  - Left Indent .75

  - Left Indent .75

    - Left Indent 1.25

    - Left Indent 1.25

      - Left Indent 1.75

      - Left Indent 1.75

Tables

The example tables below MUST be copied and pasted into the document and adjusted as needed.

Table Alignment: Centered on page

Table Width: 6.75” (maximum)

Table Caption: Centered above table, Verdana 10pt Bold, Keep with Next pagination

Cell Margins: R/L: 0.08”; Top: 0”; Bottom: 0.04”

Table Text Wrapping: AroundPositioning Bottom 0.2”

Header Row: Bold, repeats every page

To update the Table Number to reflect the correct section it was copied into, highlight the table number, right-click on the number, and select Update Field.

Table 7-1 Example Table with 3 Headings (3 columns)

Table 7-2 Example Table with 4 Headings (4 columns)

Figures

Figures are typically inserted between paragraphs or are placed at the end of a numbered section. Figure numbers should update automatically as they are inserted into the document, but if needed, they can be manually updated. To do so, highlight the figure number, right-click on the number, and select Update Field.

Insert figures on their own line using the following Layout settings:

Text Wrapping: Top and bottom

Size: Use Absolute settings with 6.75” maximum width

Position

  - Horizontal: Alignment Centered relative to column

  - Vertical: Absolute position 0” below Line

  - Options: Check Move object with text and Lock anchor

Add captions once the figures are properly placed and sized. If figure size or position is adjusted later, captions may need to be redone for proper spacing between the caption and figure. Use the following Caption settings:

Label: Figure

Position: Below selected item

Style: Caption (Verdana 10pt, bold)

Alt Text: Add a brief sentence to describe the diagram for someone who is blind or low vision. Example Alt text has been added to the figures below.

Copyrighted figure from an external source: Permission must be granted by the owner and may require a “used with permission” statement.

Store source files on NENA Workspace: Upload source files for all diagrams, images, and illustrations (e.g., Visio, Illustrator, PowerPoint) into NENA Workspace for future editing.

Figure 7-1 Effect of Geocoding Addresses against Actual Address Ranges

Figure 7-2 NENA i3 NG9-1-1 Reference Architecture

Cross-References—How to Jump Between the Citations

Use <Ctrl+click> on a cross-reference to jump to the cited reference in the References section or to the targeted section, table, or figure Cross-References – How to Jump Between in the document.

To return to the clicked cross-reference, use <Alt+Left Arrow key>.

Cross-References—How to Add

To add a Cross-reference to a cited document, web page, or other cited item listed in the References section, use these steps to insert the Reference number:

Insert the curser after the cited reference in the text and on the References tab, select Cross-reference.

To add the cited reference number, in the Cross-reference pop-up box, set Reference type: to Numbered item, and set Insert reference to: to Paragraph number.

Scroll through the For which numbered item: list and select the appropriate Reference that is being cited.

Select the Insert button.

Change the font color of the inserted reference, including the brackets, to Blue.

To add a Cross-reference to a cited section in the document, use these steps to insert the number and title of the section:

Insert the curser where you want to cite the section in the text, and on the References tab, select Cross-reference.

In the Cross-reference pop-up box, set Reference type: to Heading, and set Insert reference to: to Heading Number.

Scroll through the For which heading: list and select the appropriate section heading that is being cited.

Select the Insert button.

Add a space to separate the section heading number from the section heading title.

In the Cross-reference pop-up box, set Insert reference to: to Heading text.

In the For which heading: list, select the appropriate section heading that is being cited.

Select the Insert button.

Change the font color of the inserted number and title of the section to Blue.

To add a Cross-reference to a cited table in the document, use these steps to insert the number and title of the table:

Insert the curser where you want to cite the table in the text, and on the References tab, select Cross-reference.

In the Cross-reference pop-up box, set Reference type: to Table, and set Insert reference to: to Entire caption.

Scroll through the For which caption: list and select the appropriate table that is being cited.

Select the Insert button.

Change the font color of the inserted table caption to Blue.

To add a Cross-reference to a cited figure in the document, use these steps to insert the number and title of the figure:

Insert the curser where you want to cite the figure in the text, and on the References tab, select Cross-reference.

In the Cross-reference pop-up box, set Reference type: to Figure, and set Insert reference to: to Entire caption.

Scroll through the For which caption: list and select the appropriate figure that is being cited.

Select the Insert button.

Change the font color of the inserted figure caption to Blue.

Cross-References—How to Update

When cited references are added or removed from the References section, all cross-references in the document need to be updated. Similarly, when sections are added or removed, section numbers will change and all cross-references to sections, tables, and figures will need to be updated. These can be updated individually or all at once.

To update all cross-references at once, select the entire document <Ctrl+A>, then press F9. It is important to confirm that all cross-references were updated and go to the correct cited reference, section, table, or figure by selecting each updated cross-reference <Ctrl+click>.

To update an individual cross-reference, right-click on the specific cross-reference and select Update Field. Confirm the cross-reference was updated and goes to the correct cited reference, section, table, or figure by selecting on the updated cross-reference <Ctrl+click>.

Navigation Shortcuts

Use the Navigation Pane to quickly jump to a section or specific text in the document rather than scrolling through the pages.

On the View tab, check the Navigation Pane box to display it.

Search for a section:

  - Headings: Click on a section title to jump to that section.

  - Pages: Scroll through page thumbnail images to quickly find a table, image, or other content when a section title is unknown and a visual clue is needed. Click on the thumbnail image of interest to jump to that page.

Search for a word or phrase: Type the text in the Search document at the top of the Navigation Pane.

  - Headings: Section titles containing the typed text are highlighted in yellow.

  - Pages: Thumbnail images are displayed for only those pages containing the typed text.

  - Results: Each individual search result is displayed with some of its surrounding text.

Check Accessibility

Use Microsoft’s Accessibility Assistant to identify and fix common accessibility issues that make it difficult for blind or low vision people. The Status Bar at the bottom of the window will display Accessibility: Investigate if there is an accessibility issue.

On the Review tab, select Check Accessibility to open the Accessibility pane on the right. Issues and suggested resolutions are listed in the Accessibility pane.

Hard-to-read text contrast: If the text color is hard to see. Use the styles listed in the Styles gallery as they do not cause accessibility issues.

Missing Alt Text: A short 1-2 sentence description is needed to describe what the image is for someone who is blind or has low vision.

Use of merged or split cells: Cells should not be merged or split as it makes it difficult for screen readers to convey the information.

Inclusive Language

Review the document to confirm that terms used in the document meet NENA’s Use of Inclusive Language Policy, found in the NENA Workspace Administrative Procedure, Templates & Training Material.

Example text to show footnotes with 7pt font for the footnote number and 8pt font for the footnote text.